Final Project - Analogue Communications

Fadi Sarwat Farouk – 7432

Peter Anton Naguib – 7406

Yassin Medhat Tamam – 7574

Zeyad Ahmed Fathalla – 7621

Yamen Mohamed Saad – 7577

# Amplitude Modulation (DSB-TC and DSB-SC)

## Code

clear all;

close all;

[y, fs] = audioread('eric.wav');

Y = fftshift(fft(y));

f = linspace(-fs/2, fs/2, length(Y));

% Plot the spectrum

plot(f, abs(Y)/length(Y));

title('Spectrum of m');

figure();

% Apply filter

bw = 4000;

filt = ones(size(Y));

filt(f > bw|f<-bw) = 0;

y_filter = Y .* filt;

plot(f, abs(y_filter)/length(y_filter));

title('filtered signal spectrum');

% Inverse transform

y_filtered_time = ifft(ifftshift(y_filter));

% Ensure data type and scaling

y_filtered_time = real(y_filtered_time); % Ensure real values

y_filtered_time = double(y_filtered_time); % Convert to double if necessary

%% Normalize if values are outside the range [-1, 1]

max_val = max(abs(y_filtered_time));

if max_val > 1

y_filtered_time = y_filtered_time / max_val;

end

fc = 100000;

U = 0.5;

Am = max(y_filtered_time);

Ac = Am/U; %modulationindex = Am/Ac

new_fs = 5 * fc;

resampled_signal = resample(y_filtered_time, new_fs, fs);

t1 = linspace(0, length(resampled_signal) / new_fs, length(resampled_signal));

t1 = t1';

carrier = Ac .* cos(2 * pi * fc * t1);

DSB_SC = resampled_signal .* carrier;

DSB_TC = (1 + U * resampled_signal/Am ) .* carrier;

%% Plot DSB-SC spectrum

figure();

subplot(1,2,1)

DSB_SC_spectrum = fftshift(fft(DSB_SC));

f_DSB_SC =linspace(- new_fs/2,  new_fs/2, length(DSB_SC));

plot(f_DSB_SC, abs(DSB_SC_spectrum)/length(DSB_SC_spectrum));

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('DSB-SC Modulated Signal Spectrum');

% Plot DSB-TC spectrum

subplot(1,2,2)

DSB_TC_spectrum = fftshift(fft(DSB_TC));

f_DSB_TC=linspace(-new_fs/2,new_fs/2,length(DSB_TC));

plot(f_DSB_TC, abs(DSB_TC_spectrum)/length(DSB_TC_spectrum));

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('DSB-TC Modulated Signal Spectrum');

%% envelop for DSB-SC

envelope_DSB_SC = abs(hilbert(DSB_SC));

figure;

plot(t1, DSB_SC);

hold on;

plot(t1,-envelope_DSB_SC,'r-',t1, envelope_DSB_SC,'-r','Linewidth',1.5);

hold off;

title('DSB_sc with envellope');

ylim([-2 2]);

xlim([2 2.5]);

%envelop for DSB-TC

envelope_DSB_TC = abs(hilbert(DSB_TC));

figure;

plot(t1, DSB_TC);

hold on;

plot(t1, -envelope_DSB_TC ,'r-', t1, envelope_DSB_TC, '-r', 'Linewidth', 1.5);

hold off;

title('DSB_Tc with envellope');

ylim([-2 2]);

xlim([2 2.5]);

demod_DSB_SC=envelope_DSB_SC.*cos(2 * pi * fc * t1);

%% plot demodulation signal

figure;

subplot(2, 1, 1);

plot(t1, envelope_DSB_TC);

xlabel('Time');

ylabel('Amplitude');

title('Received Signal - DSB-TC');

subplot(2, 1, 2);

plot(t1, demod_DSB_SC);

xlabel('Time');

ylabel('Amplitude');

title('Received Signal - DSB-SC');

% resample the envelope DSB_SC to hear it

recived_sig_DSB_SC = resample(envelope_DSB_SC, fc, fs);

% resample the envelope DSB_TC to hear it

recived_sig_DSB_TC = resample(envelope_DSB_TC, fc, fs);

%% sound the three msgs

sounds={y_filtered_time,envelope_DSB_SC,envelope_DSB_TC};

for i = 1:length(sounds)

sound(sounds{i}, fs);

pause(10);

end

%% Coherent detection

% when noise is added to DSB_SC with SNR = 0, 10, and 30

snr_values = [0, 10, 30];

for snr_dB = snr_values

% Add noise to DSB_SC

noisy_DSB_SC = awgn(DSB_SC, snr_dB);

demodulated_noisy = noisy_DSB_SC .* cos(2*pi*fc*t1);

f = new_fs / 2 * linspace(-1,1,length(DSB_SC));

% Coherent detection with noise

demodulated_noisy = double(real(demodulated_noisy));

demodulated_noisy = fftshift(fft(demodulated_noisy));

demodulated_noisy(f >= bw | f <= -bw) = 0;

demodulated_noisy = ifft(ifftshift(demodulated_noisy));

L = length(demodulated_noisy);

% Plot received waveform and spectrum for each SNR value

figure;

subplot(2, 1, 1);

plot(t1, demodulated_noisy);

title(['Received Signal with SNR = ' num2str(snr_dB) ' dB - Time Domain']);

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2, 1, 2);

plot(f, abs(fftshift(fft(demodulated_noisy))) / L);

title(['Spectrum with SNR = ' num2str(snr_dB) ' dB']);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

demodulated_noisy_sound = resample(abs(demodulated_noisy), fs, new_fs);

sound(abs(demodulated_noisy_sound), fs);

pause(10);

end

%% coherent detection with frequency error

fc_error=100100;

for snr_dB = snr_values

% Add noise to DSB_SC

noisy_DSB_SC = awgn(DSB_SC, snr_dB);

demodulated_noisy = noisy_DSB_SC .* cos(2*pi*fc_error*t1);

f = new_fs / 2 * linspace(-1,1,length(DSB_SC));

% Coherent detection with noise

demodulated_noisy = double(real(demodulated_noisy));

demodulated_noisy = fftshift(fft(demodulated_noisy));

demodulated_noisy(f >= bw | f <= -bw) = 0;

demodulated_noisy = ifft(ifftshift(demodulated_noisy));

L = length(demodulated_noisy);

% Plot received waveform and spectrum for each SNR value

figure;

subplot(2, 1, 1);

plot(t1, demodulated_noisy);

title(['Received Signal with SNR (FE) = ' num2str(snr_dB) ' dB - Time Domain']);

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2, 1, 2);

plot(f, abs(fftshift(fft(demodulated_noisy))) / L);

title(['Spectrum with SNR (FE) = ' num2str(snr_dB) ' dB']);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

demodulated_noisy_sound = resample(abs(demodulated_noisy), fs, new_fs);

sound(abs(demodulated_noisy_sound), fs);

pause(10);

end

%% phase error

for snr_dB = snr_values

% Add noise to DSB_SC

noisy_DSB_SC = awgn(DSB_SC, snr_dB);

demodulated_noisy = noisy_DSB_SC .* cos(2*pi*fc*t1+20);

f = new_fs / 2 * linspace(-1,1,length(DSB_SC));

% Coherent detection with noise

demodulated_noisy = double(real(demodulated_noisy));

demodulated_noisy = fftshift(fft(demodulated_noisy));

demodulated_noisy(f >= bw | f <= -bw) = 0;

demodulated_noisy = ifft(ifftshift(demodulated_noisy));

L = length(demodulated_noisy);

% Plot received waveform and spectrum for each SNR value

figure;

subplot(2, 1, 1);

plot(t1, demodulated_noisy);

title(['Received Signal with SNR (PE) = ' num2str(snr_dB) ' dB - Time Domain']);

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2, 1, 2);

plot(f, abs(fftshift(fft(demodulated_noisy))) / L);

title(['Spectrum with SNR (PE) = ' num2str(snr_dB) ' dB']);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

demodulated_noisy_sound = resample(abs(demodulated_noisy), fs, new_fs);

sound(abs(demodulated_noisy_sound), fs);

pause(10);

end

## Outputs

## Questions

1) What observation can you make of this or which type of modulation the envelope detector can be used with?

In the DSB-TC the envelop detection worked and it was clear since there is no phase reversals in the DSB-SC the envelop detection wasn't optimal since the modulation index is critical and the coherent detection for it worked better.

2) Do you have a name for this phenomenon?

Frequency Offset

Top of Form

Bottom of Form

# Single Side Band (SSB)

## Code

%% 1-2. Same as DSB

[y, fs] = audioread('eric.wav');

L = length(y);

Y = fftshift(fft(y));

f = linspace(-fs/2, fs/2, L);

% Plot the spectrum

figure;

subplot(2, 1, 1);

plot(f, abs(Y) / L);

title('Original Spectrum of m');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% Apply filter

bw = 4000;

Y(f >= bw | f <= -bw) = 0;

subplot(2, 1, 2);

plot(f, abs(Y) / L);

title('Filtered Spectrum of m');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

%% 3. Filtered signal in time domain (Inverse transform)

y_filtered_time = ifft(ifftshift(Y));

t1 = linspace(0, length(y_filtered_time) / fs, length(y_filtered_time));

t1 = t1';

y_filtered_time = real(double(y_filtered_time));

figure; plot(t1, y_filtered_time);

title('Filtered Signal of m Time Domain');

xlabel('Time (s)');

ylabel('Amplitude');

fc = 100000;

U = 0.5;

Am = max(y_filtered_time);

Ac = Am/U; % modulationindex = Am/Ac

new_fs = 5 * fc;

%% 4. Plot DSB-SC spectrum

message_for_sound = resample(y_filtered_time, new_fs, fs);

% sound(abs(message_for_sound), fs);

message = resample(y_filtered_time, new_fs, fs);

t1 = linspace(0, length(message) / new_fs, length(message));

t1 = t1';

L = length(message);

carrier = Ac .* cos(2*pi*fc*t1);

DSB_SC = message .* carrier;

DSB_SC_spectrum = fftshift(fft(DSB_SC));

f_DSB_SC = new_fs/2 * linspace(-1, 1, length(DSB_SC));

figure;

subplot(2, 1, 1);

plot(f_DSB_SC, abs(DSB_SC_spectrum) / L);

title('DSB-SC Modulated Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% 5. Obtain SSB_LSB

SSB_LSB = DSB_SC;

L = length(SSB_LSB);

f = new_fs / 2 * linspace(-1, 1, L);

F = fftshift(fft(SSB_LSB));

F(f>=fc | f<=-fc) = 0;

SSB_LSB = ifft(ifftshift(F));

% Plot SSB_LSB spectrum

subplot(2, 1, 2);

plot(f, abs(F) / L);

title('SSB LSB Modulated Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

%% 6. Coherent detection with no noise interference

demodulated_signal_ideal = SSB_LSB .* cos(2*pi*fc*t1);

demodulated_signal_ideal_sound = resample(abs(demodulated_signal_ideal), fs, new_fs);

sound(abs(demodulated_signal_ideal_sound), fs);

demodulated_signal_ideal = fftshift(fft(demodulated_signal_ideal));

demodulated_signal_ideal(f >= fc | f <= -fc) = 0;

demodulated_signal_ideal = ifft(ifftshift(demodulated_signal_ideal));

% Plot received waveform and spectrum

subplot(2, 1, 1);

plot(t1, demodulated_signal_ideal);

title('Received Signal (CD) in Time domain');

xlabel('Time (s)');

ylabel('Amplitude');

% Spectrum of received signal

L = length(demodulated_signal_ideal);

Y_demodulated_ideal = fftshift(fft(demodulated_signal_ideal));

f_demodulated_ideal = linspace(-new_fs/2, new_fs/2, L);

subplot(2, 1, 2);

plot(f_demodulated_ideal, abs(Y_demodulated_ideal) / L);

title('Spectrum of Received Signal (CD)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([-10000 10000]);

%% 7. Repeat steps 5 and 6 with Butterworth filter

[b, a] = butter(4, fc / (new_fs / 2), 'low');

SSB_LSB_butter = filtfilt(b, a, demodulated_signal_ideal);

L = length(SSB_LSB_butter);

% Plot the spectrum of SSB-LSB signal using Butterworth filter

Y_SSB_LSB_butter = fftshift(fft(SSB_LSB_butter));

f_SSB_LSB_butter = linspace(-fs/2, fs/2, length(Y_SSB_LSB_butter));

figure;

subplot(3, 1, 1);

plot(f_SSB_LSB_butter, abs(Y_SSB_LSB_butter) / L);

title('Spectrum of SSB-LSB Signal (Butterworth Filter)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% Coherent detection with Butterworth filter

demodulated_signal_butter = SSB_LSB_butter .* carrier;

t = linspace(0, length(demodulated_signal_ideal) / fs, length(demodulated_signal_ideal));

% Plot received waveform and spectrum

subplot(3, 1, 2);

plot(t, demodulated_signal_butter);

title('Received Signal (Coherent Detection with Butterworth) - Time Domain');

xlabel('Time (s)');

ylabel('Amplitude');

% Spectrum of received signal with Butterworth filter

Y_demodulated_butter = fftshift(fft(demodulated_signal_butter));

f_demodulated_butter = linspace(-fs/2, fs/2, length(Y_demodulated_butter));

subplot(3, 1, 3);

plot(f_demodulated_butter, abs(Y_demodulated_butter) / L);

title('Spectrum of Received Signal (Coherent Detection with Butterworth)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

%% 8. For the ideal filter case, get the received signal again with noise

% when noise is added to SSB-SC with SNR = 0, 10, and 30

snr_values = [0, 10, 30];

for snr_dB = snr_values

% Add noise to SSB-SC

noisy_SSB_SC = awgn(SSB_LSB, snr_dB);

demodulated_noisy = noisy_SSB_SC .* cos(2*pi*fc*t1);

% Coherent detection with noise

demodulated_noisy = double(real(demodulated_noisy));

demodulated_noisy = fftshift(fft(demodulated_noisy));

demodulated_noisy(f >= bw | f <= -bw) = 0;

demodulated_noisy = ifft(ifftshift(demodulated_noisy));

f = new_fs / 2 * linspace(-1,1,L);

L = length(demodulated_noisy);

% Plot received waveform and spectrum for each SNR value

figure;

subplot(2, 1, 1);

plot(t1, demodulated_noisy);

title(['Received Signal with SNR = ' num2str(snr_dB) ' dB - Time Domain']);

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2, 1, 2);

plot(f, abs(fftshift(fft(demodulated_noisy))) / L);

title(['Spectrum with SNR = ' num2str(snr_dB) ' dB']);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

demodulated_noisy_sound = resample(abs(demodulated_noisy), fs, new_fs);

sound(abs(demodulated_noisy_sound), fs);

end

%% 9. For the ideal filter case, generate a SSB-TC

SSB_TC = carrier + SSB_LSB;

L = length(SSB_TC);

% Spectrum of received signal

Y_SSB_TC = fftshift(fft(SSB_TC));

f_SSB_TC = linspace(-new_fs/2, new_fs/2, length(Y_SSB_TC));

figure;

plot(f_SSB_TC, abs(Y_SSB_TC) / L);

title('Spectrum of Received SSB-TC Signal');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% Envelope detection

envelope_SSB_TC = abs(hilbert(SSB_TC));

% Plot received waveform

figure;

plot(t1, SSB_TC, 'b', t1, -envelope_SSB_TC, 'r', t1, envelope_SSB_TC, 'r', 'Linewidth', 1.5);

title('Received Message with Envelope Detection');

xlabel('Time (s)');

ylabel('Amplitude');

legend('SSB-TC', 'Envelope');

ylim([-5 5]);

xlim([3 3.5]);

envelope_SSB_TC = resample(envelope_SSB_TC, fs, new_fs);

sound(abs(envelope_SSB_TC), fs);

## Outputs

# Frequency Modulation (FM)
Code

% 1. Read the audio file and find the spectrum

[audio, Fs] = audioread('eric.wav');

N = length(audio);

t = (0:N-1) / Fs;

% Calculate the spectrum

audio_spectrum_shifted = fftshift(fft(audio));

% Plot the spectrum

figure;

plot(linspace(-Fs/2, Fs/2, N), abs(audio_spectrum_shifted));

title('Original Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% 2. Ideal filter to remove frequencies above 4 KHz

cutoff_frequency = 4000;

filter = (abs(linspace(-Fs/2, Fs/2, N)) <= cutoff_frequency);

audio_spectrum_filtered = audio_spectrum_shifted .* filter';

% 3. Obtain filtered signal in time and frequency domain

audio_filtered =real(ifft(ifftshift(audio_spectrum_filtered)));

% Plot the filtered spectrum

figure;

plot(linspace(-Fs/2, Fs/2, N), abs(audio_spectrum_filtered));

title('Filtered Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% Plot the filtered signal in time domain

figure;

plot(t, audio_filtered);

title('Filtered Signal in Time Domain');

xlabel('Time (s)');

ylabel('Amplitude');

% 4. Sound the filtered audio signal

sound(abs(audio_filtered), Fs);

% 5. Generate NBFM signal

kf = 0.2/(2*pi*max(abs(cumsum(audio_filtered)))./Fs);

Ac = 1;

Fc = 100000; % Carrier frequency

resampled_audio = resample(audio_filtered, 5*Fc, Fs);

Fs_nbfm = 5 * Fc; % Sampling frequency for NBFM

%calculate time vector

tstart = 0;

tend = tstart + length(resampled_audio) / Fs_nbfm;

t1 = linspace(tstart, tend, length(resampled_audio));

t1 = t1';

%FM modulated signal

NBFM_signal = Ac * cos(2*pi*Fc*t1 + 2*pi*kf*cumsum(resampled_audio)./Fs_nbfm);

% Plot the NBFM spectrum

L = length(NBFM_signal);

NBFM_spectrum_shifted = real(fftshift(fft(NBFM_signal)));

f = Fs/2*linspace(-1,1,L);

figure;

plot(f, NBFM_spectrum_shifted)

title('NBFM Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% 6. Condition for NBFM: Frequency deviation (delta_f) should be much smaller than the message bandwidth (B)

B = cutoff_frequency;

delta_f = 75; % Frequency deviation

condition = delta_f < B;

disp(['Condition for NBFM: ', num2str(condition)]);

% 7. Demodulate NBFM signal

% Discriminator

dy = diff(NBFM_signal);

dy = [0; dy];

% envelope detector

demodulated_NBFM = abs(hilbert(dy)) -  mean(abs(hilbert(dy)));

% Plot the time-domain signal

figure;

plot(t1,demodulated_NBFM);

title('Demodulated NBFM Time-Domain Signal');

xlabel('Time (s)');

ylabel('Amplitude');

ylim([-2*10^-4 2*10^-4]);

## Outputs

|  | NBFM Zoomed in |
| --- | --- |

## Questions

1) What can you make out of the resulting plot?

We realized that the Bandwidth=2fm since beta is very small (condition for NBFM).

2) what is the condition we needed to achieve NBFM?

To acheive NBFM we need to satisfy the condition in this equation BW= 2(beta+1) * Fm that beta <= 0.2